=========================

LG Electronics GH22NS40

Firmware Version NL02

=========================

Never update the firmware if the problem you’re having is the drive

isn't seen by windows, won't write with the included software, or other

interface related problems, unless the firmware specifically states it

fixes the problem you are having. Firmware updates normally don't fix these

types of problems. Trying to update the firmware when there are interface

address these problems can damage the drive's firmware. An error when

updating the firmware is usually caused by a problem with the drive

interface or there is more than one drive connected to that IDE channel.

------------------

Version History

------------------

NL01

- Improve Writing Strategy

NL02

- Support Windows 7

--------------------

Supported Drives

--------------------

This firmware upgrade is for LG Branded drives. This upgrade is NOT

designed for drives manufactured for HP, IBM, Compaq, Dell or any

other OEM (Original Equipment Manufacturer) customers, please contact

the OEM for all firmware updates and support for your OEM drive.

------------------------

Upgrade Instructions

------------------------

This firmware software only works in windows mode with Windows

2000/XP/Vista. If you encounter problems in flashing the firmware or have

any questions, please contact our LG customer support line toll-free at

1-800-243-0000.

1. If you haven't already, download the firmware file and extract the zip

file.

2. Connect the drive to the secondary IDE channel as a master drive with

no other devices connected to that channel (Ignore if it's an SATA drive).

Start Windows and verify it can recognize the drive.

3. Close all other programs including: screen saver, and /or antivirus.

Do not open any programs or start any system functions during the

update as they can interfere with data transfer and damage the drive.

4. Open the firmware program.

5. Click the update button and do not touch or interfere with the drive

while the firmware is being upgraded. IMPORTANT - DO NOT INTERFERE WITH

THE DRIVE DURING THIS PROCESS. If an error occurs during the upgrade

process, or the upgrade process is cancelled during this process, damage

to the drive can occur.

6. When the upgrade is complete the screen should say FIRMWARE UPDATE

SUCCESSFUL. Re-boot your PC.

7. Enjoy!

If you experience any problems or have any questions, please contact LG

customer support at 1-800-243-0000.